|  | ELECTRICAL WORK ORDER Alder Residence — Portland, OR |
| --- | --- |

| Customer | Casey Alder |
| --- | --- |
| Address | 1234 SE Alder St, Portland, OR 97215 |
| Phone | (503) 555-0136 |
| Questions on site | Kyle — (503) 555-0117 |
| Job # | 2026-0714 |
| Install Dates | ___________________________ |
| Crew Lead | ___________________________ |

⚠  SAFETY FLAGS: LEAD PAINT — 1962 home, lead-safe (RRP) practices on any paint disturbance

Scope of Work

| Task | Details | Location |
| --- | --- | --- |
| New 240V circuit for heat pump | 25A 2-pole breaker, 10/2 from panel to east-wall disconnect, whip to ODU | Panel (garage) to east wall |

Equipment Arriving

| Item | Model | Qty |
| --- | --- | --- |
| Outdoor unit | 24k cold-climate ductless ODU | 1 |
| Wall heads | 12k (living) + 9k (bedroom) | 2 |
| Bath fan | 110 CFM ENERGY STAR, humidistat | 1 |
| Range hood | 300 CFM ducted | 1 |
| Disconnect + whip | 60A non-fused | 1 |

Burch QA — Electrical

☐  New circuits sized correctly for HP + aux + HPWH (Q159)

☐  All major electrical work permitted and closed out (Q160)

☐  Panel size sufficient for load — 100A vs 200A verified (Q161)

☐  Breakers labeled at the panel

Permits

☐  Permits closed out for EACH inspected installed measure (Q162)

☐  Permit finals scheduled BEFORE requesting the program QA inspection

☐  Inspection cards / permit numbers photographed for the job file

Notes for Crew

•  Homeowner is on video calls in the back office — knock at the front door, headphones mean go around

•  Keep the driveway clear of material staging after 4pm (street sweeping Tuesday)

•  Cat is friendly and sneaky — check for her before closing the attic hatch

Reference Photos

| Panel in garage — ODU circuit lands in the spare spaces, lower right | East wall ODU location — pad here, lineset penetration sealed after ducts run |
| --- | --- |

Resilient Retrofits  ·  CCB# 245539  ·  Prepared by: Kyle Kanooth