|  | WEATHERIZATION WORK ORDER Alder Residence — Portland, OR |
| --- | --- |

| Customer | Casey Alder |
| --- | --- |
| Address | 1234 SE Alder St, Portland, OR 97215 |
| Phone | (503) 555-0136 |
| Questions on site | Kyle — (503) 555-0117 |
| Job # | 2026-0714 |
| Install Dates | ___________________________ |
| Crew Lead | ___________________________ |

⚠  SAFETY FLAGS: LEAD PAINT — 1962 home, lead-safe (RRP) practices on any paint disturbance

Scope of Work

| Task | Details | Location |
| --- | --- | --- |
| Attic air seal | Full ceiling plane: top plates, can lights, penetrations, flue dam; hold the hood + bath-fan duct spots Day 1 and close them after HVAC runs ducts Day 2 | Attic |
| Attic prep + insulation | Baffles at every eave bay, hatch dam + weatherstrip, 3 depth markers, then blow R-49 cellulose over 940 sf | Attic |

Equipment Arriving

| Item | Model | Qty |
| --- | --- | --- |
| Outdoor unit | 24k cold-climate ductless ODU | 1 |
| Wall heads | 12k (living) + 9k (bedroom) | 2 |
| Bath fan | 110 CFM ENERGY STAR, humidistat | 1 |
| Range hood | 300 CFM ducted | 1 |
| Disconnect + whip | 60A non-fused | 1 |

Burch QA — Air Sealing

☐  All MEP penetrations sealed — attic to conditioned space (Q6)

☐  All MEP penetrations sealed — crawl to conditioned space (Q7)

☐  Gaps around chimneys/gas vents sealed with high-temp caulk/foam (Q8)

☐  Drywall sealed to top plates (Q11)

☐  Weatherstripping on all exterior door openings (Q10)

☐  Pre AND post blower door test conducted and recorded (Q13)

Burch QA — Attic Insulation

☐  Mold/water/pest damage mitigated before insulating (Q15)

☐  Attic ventilation at 1:300 net free area with bug screens on vents (Q16)

☐  Attic access lid insulated R-38+ and weatherstripped airtight (Q17)

☐  All ductwork sealed mastic/UL-181, insulated R-8, supported (Q18)

☐  Knee/skylight walls insulated R-21+ with attic-side air barrier (Q21)

☐  Insulation dams around chimneys/flues in heat-safe materials (Q22)

☐  Non-airtight recessed cans boxed over, seams sealed (Q23)

☐  Attic insulated to R-49, Grade 1 installation (Q25)

☐  Insulation baffles at all bird blocks (Q26)

Burch QA — ASHRAE 62.2 Ventilation

☐  Whole-house ventilation meets the ASHRAE 62.2 calculation (Q44)

☐  Kill switch acceptably located for continuous fans (Q46)

☐  Controls labeled appropriately (Q48)

☐  Operation/maintenance instructions provided to occupant (Q49)

☐  Whole-house exhaust fan rated 1 sone or quieter (Q50)

Burch QA — Safety & Critical Repairs

☐  Radon test at end of project — result 4 pCi/L or lower — when air sealing is in scope (Q133)

☐  Smoke and CO monitors installed to code (Q136)

☐  Asbestos handled per Oregon DEQ when present (Q131-132)

☐  Lead paint: EPA RRP compliance on pre-1978 homes when paint is disturbed (Q137)

Permits

☐  Permits closed out for EACH inspected installed measure (Q162)

☐  Permit finals scheduled BEFORE requesting the program QA inspection

☐  Inspection cards / permit numbers photographed for the job file

Notes for Crew

•  Homeowner is on video calls in the back office — knock at the front door, headphones mean go around

•  Keep the driveway clear of material staging after 4pm (street sweeping Tuesday)

•  Cat is friendly and sneaky — check for her before closing the attic hatch

Reference Photos

| Attic from hatch — vacuum out, air-seal, baffles at every eave bay | Hall bath fan location — duct out the roof, first 3 ft straight |
| --- | --- |
| Range wall — hood ducts through the upper cabinet, out the wall cap |  |

Resilient Retrofits  ·  CCB# 245539  ·  Prepared by: Kyle Kanooth